Document No. MIM-TR-EPS-002 Document Author Name Responsible Engineer Name Date 2019.12.12

EPS Specifications

Description of MIMAN EPS General Specifications

Revision History Date; Version; Author; Description. 2019.12.12; 1.0; 전승권; 초안 작성. 2022.03.07; -; 전승권; 내용 수정. 2022.03.08; 2.0; 유성엽; 그림 추가.

List of Abbreviations and Acronyms

Contents; Description. SOC; State of Charge. DOD; Depth of Discharge. PCM; Power Conditioning Module. PDM; Power Distribution Module. PCDU; Power Conditioning and Distribution Unit. MPPT; Maximum Power Point Tracker. BCR; Battery Charge Regulator. SEPIC; Single Ended Primary Inductor Converter. LTAN; Local Time Ascending Node.

Contents

1	MIMAN EPS H/W Configuration	5

1.1	Solar Cell	5

1.1.1	H/W Electrical Specification	5

1.2	PCDU	6

1.2.1	H/W Features	6

1.2.1.1	PCM	6

1.2.1.2	PDM	7

1.2.1.3	BCR	7

1.3	Battery	7

1.3.1	H/W Electrical Specification	7

1.4	MIMAN EPS Interface	7

2	MIMAN Power consumption	7

2.1	MIMAN H/W Power consumption	7

2.2	H/W Voltage BUS	7

2.3	PCM & PDM Load	7

3	MIMAN EPS Power generation	8

3.1	Solar panel configuration	8

3.1.1	Solar panel Electrical Specification	8

4	Reference	8

5	Appendix	8

- MIMAN EPS H/W Configuration

MIMAN EPS는 Solar Cell, PCDU 그리고 배터리로 구성된다.

  - Solar Cell

MIMAN EPS에서는 AZURE SPACE사의 “TJ Solar Cell 3G30C - Advanced”를 사용한다.

그림 1. TJ Solar Cell 3G30C - Advanced

    - H/W Electrical specification

| [Electrical Data] | BOL | 2,5E14 | 5E14 |  | 1E15 |
| --- | --- | --- | --- | --- | --- |
| Average Open Circuit Voc [mV] | 2700 | 2616 | 2564 |  | 2522 |
| Average Short Circuit Isc [mA] | 457.0 | 455.5 | 451.6 |  | 440.9 |
| Voltage at max. Power Vmp [mV] | 2411 | 2345 | 2290 |  | 2246 |
| Current at max. Power Imp [mA] | 442.8 | 442.1 | 439.8 |  | 427.5 |
| Average Efficiency ŋbare (1367 W/m2) [%] | 29.5 | 28.6 | 27.8 |  | 26.5 |
| Average Efficiency ŋbare (1353 W/m2) [%] | 29.8 | 28.9 | 28.1 |  | 26.8 |
|  |  |  | @fluence 1MeV [e/cm2] |  |  |
| 표 1. 전기적 특성 - 3G30C – Advanced |  |  |  |  |  |
| [Acceptance Values] |  |  |  |  |  |
| Voltage Vop [mV] |  |  |  | 2350 |  |
| Min. average current Iop avg @ Vop [mA] |  |  |  | 444 |  |
| Min. individual current Iop min @ Vop [mA] |  |  |  | 417 |  |
| 표 2. 허용 수치 - 3G30C – Advanced |  |  |  |  |  |

  - PCDU

MIMAN EPS에서는 ISIS 사의 “Compact EPS version 2 - CEPSv2”를 사용한다

그림 2. ISIS CEPSv2

    - H/W Characteristics

- Maximum power point tracking (MPPT) 기능

- H/W 과전압, 과전류 방지 기능 탑재

- 방사선 내구성 향상을 위한 FRAM 기반 MCU

- I2C 통신 사용

- 추가 보드 장착을 통한 전기적 연결 확장 가능

그림 3. ISIS CEPS PCDU Top Connector Pinout

그림 4.  ISIS CEPS PCDU Bottom Connector Pinout

      - PCM

- 총 4개의 전압 영역을 제공한다: 3.3 V @ 4 A, 5 V @ 4 A, 12 V(user selectable) @ 4A, Vbat (12.8 – 16 V) @ 4A (VD0: Vbat/ VD1: 3.3 V/ VD2: 5 V/ VD3: 기본 5 V(최대 12 V 조정 가능)/ VD4: 기본 3.3 V(최대 12 V 조정 가능)

- Mother Board – VD0 & VD1 & VD2, Daughter Board – VD0 & VD3 & VD4

- BCR

그림 5. Chain efficiency – 태양 전지판-배터리 Chain Efficiency – type B & C

      - PDM

- 개별 출력 제어 스위치 제공_모든 출력 채널에 대해 제어 스위치를 제공

(총 16의 출력 채널과 그에 따른 제어 스위치 제공)

    - H/W Electrical specification

| Electrical Characteristics |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Static (Idle) consumption IU mainboard |  |  |  |  | 90 | mW |  |
| Power output IU mainboard (type A, B and C) | VD0 | Number of output channels | Type A and B |  | 1 | - |
|  |  |  | Type C |  | 1+2 |  |
|  |  | Output voltage | Type A and B |  | 6.5 – 8.0 | V |
|  |  |  | Type C |  | 12.8 – 16.0 |  |
|  | VD1 | Number of output channels |  |  | 4 | - |
|  |  | Output voltage |  |  | 5 | V |
|  |  | Output max. current per channel |  |  | 2 | A |
|  |  | Output max. current total |  |  | 4 | A |
|  | VD2 | Number of output channels |  |  | 4 | - |
|  |  | Output voltage |  |  | 3.3 | V |
|  |  | Output max. current per channel |  |  | 2 | A |
|  |  | Output max. current total |  |  | 4 | A |
| Power output Daughterboard (type C only) | VD3 | Number of output channels |  |  | 4 | - |
|  |  | Output voltage |  |  | Custom (12 V max) Default: 5V | V |
|  |  | Output max. current per channel |  |  | 2 | A |
|  |  | Output max. current total |  |  | 4 | A |
|  | VD4 | Number of output channels |  |  | 1 | - |
|  |  | Output voltage |  |  | Custom (12 V max) Default: 3.3 V | V |
|  |  | Output max. current per channel |  |  | 2 | A |
|  |  | Output max. current total |  |  | 2 | A |
| Solar panel input | Number of MPPTs inputs |  |  | Type A and B | 3 | - |
|  |  |  |  | Type C | 3+2 |  |
|  | Maximum input voltage |  |  | Type A and B | 8.0 | V |
|  |  |  |  | Type C | 16. |  |
|  | Minimum input voltage to start MPPT |  |  |  | 3.5 | V |
|  | Minimum input voltage if another channel’s voltage is sufficient to start MPPTs |  |  |  | 2.0 | V |
|  | Maximum input current per channel |  |  |  | 2 | A |

표 4. Electrical Characteristics: iEPS Electrical Power System – CEPS

    - H/W Block Diagram

그림 6. Block Diagram: CEPS – ISIS

  - Battery

MIMAN EPS는 ISIS에서 제조된 배터리(CEPS에서 배터리)를 사용한다.

그림 9. 4-Cell Battery Pack (ISIS)

    - H/W Feature

- 4 직렬 배터리 팩
- 45Wh 용량 (각 베터리 셀 용량: 3.2Ah)

- 히터 제공

    - H/W Electrical Specification

| Configurations | Number of cells | 3200 mAh cell (Nominal) |
| --- | --- | --- |
| 4S | 4 | 45 Wh 12.8 - 16 V 3.2 Ah |

표 5. CEPS 배터리 구성

| Battery | Cell | Min., Nom., Max. Voltage (absolute maximum) |  | 2.5, 3.6, 4.2 | V |
| --- | --- | --- | --- | --- | --- |
|  |  | Nominal Capacity |  | 3.2 | Ah |
|  | Pack | Configuration | Type A | 2 in series | - |
|  |  |  | Type B and C | 4 in series |  |
|  |  | Operating Voltage (hardware limits) | Type A | 5.5 – 8.0 | V |
|  |  |  | Type B and C | 10.9 – 16.0 |  |
|  |  | Operating Voltage (software limits) | Type A | 6.5 – 8.0 | V |
|  |  |  | Type B and C | 12.8 – 16.0 |  |
|  |  | Max Input Current |  | 1.5 | A |
|  |  | Max Output Current |  | 4 | A |
|  |  | Maximum heater power | Type A | 1.2 | W |
|  |  |  | Type B and C | 2.5 |  |

표 6. ISIS – CEPS 4-cell Battery pack

  - MIMAN EPS Interface

그림 10. MIMAN EPS Electrical Interface

- MIMAN Power consumption

  - MIMAN H/W Power consumption

각 서브시스템 하드웨어 별 소비 전력

| Subsystem | Component | Operation Description | Operation Power(W) |
| --- | --- | --- | --- |
| PAYLOAD | Multi spectral imager | Imaging | 5.1 |
| GNC | GPS RX | L1 Typ. | 0.9 |
|  | GPS Ant. | 15 mA Max. (10 mA Typ.) | 0.0495 |
| ADCS | ADCS Board / IFB | + MMT, IMU | 0.806 |
|  | Fine Sun sensor | Max. | 0.0759 |
|  | Coarse Sun Sensor | - | 0.75 |
|  | Analog to Digital Conv. | - | 0.002475 |
|  | MTQ X 3EA | 0.13~0.24 Am2 | 1.5 |
|  | Reaction Wheel Assembly (RWA) X 3EA | Max. 6000 rpm (16V + 3.3V) | 7.125 |
|  |  | 2000 rpm (16V + 3.3V) | 0.615 |
|  |  | zero rpm (3.3V) | 0.15 |
|  | Star Tracker | Nominal. | 1 |
| TT&C | UHF TRX | RX | 0.396 |
|  |  | TX | 2.805 |
|  | UHF Ant. | Idle (I2C) | 0.01 |
|  |  | Deployment | 5.6 |
|  | S-band TX. | TX active (max) | 5 |
|  |  | Off | 0.61 |
| Deployment | Solar Array | Solar panel | 2 |
| CNDH | OBC/IFB | Gigabit Ethernet [OBC] | 1 |
|  |  | Max. | 0.8 |
| EPS | PCDU | 3V3 / 5V / 12V / Vbat BUS (Idle) | 0.09 |
|  | Battery | I2C | 0 |
|  |  | Heater | 2.5 |

표 7. MIMAN H/W Power consumption

  - H/W Voltage BUS

| Subsystem | Component | Product | Operation Voltage(V) |
| --- | --- | --- | --- |
| PAYLOAD | Multi spectral imager | B3420 | 12 |
| GNC | GPS RX | OEM719 | 3.3 |
|  | GPS Ant. | TW1421 | 3.3 |
| ADCS | ADCS Board / IFB | STM32H753xI | 3.3 |
|  | Fine Sun sensor | nanoSSOC-D60 | 3.3 |
|  | Coarse Sun Sensor | SFH 2430 | 3.3 |
|  | Analog to Digital Conv. | AEC-Q100 | 3.3 |
|  | MTQ X 3EA | 2 X Rod S, 1 X Air | 5 |
|  | Reaction Wheel Assembly (RWA) X 3EA | Cubewheel M | 5 & 16 |
|  | Star Tracker | KU Leuven Star Tracker | 5 |
| TT&C | UHF TRX | NanoCom AX100 | 3.3 |
|  | UHF Ant. | UHF ANTENNA III | 5 |
|  | S-band TX. | PULSAR-STX | 5 |
| Deployment | Solar Array | Pogo-pin | 16 |
| CNDH | OBC/IFB | STM32H753xI | 3.3 |
| EPS | PCDU | iEPS Type A |  |
|  | Battery | ISIS 12.8Ah(45Wh) |  |

표 8. MIMAN H/W Voltage BUS

  - PCM Load

| Voltage BUS | BUS limit | LEOP (Initialization) | LEOP (ANT Deployment) | LEOP (Detumbling) | Commissioning |
| --- | --- | --- | --- | --- | --- |
| 12.8~16V Bus[W] | 108 | 0.00 | 0.00 | 0.00 | 8.10 |
| 8 V-6 V Bus [W] | 19.56 | 0.69 | 6.91 | 2.36 | 1.80 |
| 5 V Bus [W] | 13.16 | 6.43 | 6.43 | 6.43 | 7.98 |
| 3.3 V Bus [W] | 11.5 | 0.00 | 0.00 | 0.00 | 5.67 |
| Normal | Normal (SA Deployment) | Communication | Mission | Safe Hold | Detumbling |
| 8.10 | 8.10 | 8.10 | 8.10 | 8.10 | 0.00 |
| 2.36 | 2.36 | 2.36 | 3.47 | 2.36 | 2.36 |
| 7.98 | 10.21 | 7.98 | 9.09 | 6.93 | 6.43 |
| 0.00 | 0.00 | 0.00 | 5.67 | 0.00 | 0.00 |

표 9. MIMAN PCM Load for each Mode

- MIMAN EPS Power generation

  - Solar panel configuration & Connection

| Panel configuration | Panel | Solar Cell Configuration (Series) |
| --- | --- | --- |
| Series | Deployable Solar Panel 1(-Y-) | 6 |
| Series | Deployable Solar Panel 2(+Y) | 6 |
| Series | Body mounted Solar Panel (-Z) | 2 |
| Series | Body mounted Solar Panel (+X) | 4 |
| Series | Body mounted Solar Panel (-X) | 5 |

표 10. Solar panel configuration

그림 11. MIMAN EPS Solar panel Connection

3.1.1 Solar panel Electrical Specification

| Panel | Solar Cell Configuration | Electrical Specification (max. EOL) |
| --- | --- | --- |
| Deployable Solar Panel 1(-Y) | 3S2P | 6.74V @ 0.972A |
| Deployable Solar Panel 2(+Y) | 3S2P | 6.74V @ 0.972A |
| Body mounted Solar Panel (-Z) | 2S | 4.49V @ 0.486A |
| Body mounted Solar Panel (-X) | 5S | 8.98V @ 0.486A |
| Body mounted Solar Panel (+X) | 4S | 11.23V @ 0.486A |

표 11. Solar panel Electrical Specification

  - Power Generation

3.2.1 Sun-Pointing

그림 12. Power Generation Sun-pointing

평균 전력: 11.82W

Communication(Day), Charge, Normal 모드에서 적용된다.

3.2.2 Nadir Pointing

그림 13. Power Generation at Communication mode

평균 생산 전력: 5.5711 W

그림 14. Power Generation at Mission mode

평균 생산 전력: - W

- Reference

http://www.azurspace.com/images/006050-01-00_DB_3G30C-Advanced.pdf

https://www.cubesatshop.com/wp-content/uploads/2019/06/ISIS-EPS-Brochure-web.pdf

ISIS Compact EPS version 2-Hardware ICD

https://www.batteryspace.com/prod-specs/NCR18650B.pdf

- Appendix