Document No. MIM-TR-EPS-003 Document Author Seong-Yeop Yu Responsible Engineer Geuk-Nam Kim Date 2019.12.05

EPS Power Analysis

Electrical power budget & load analysis

Revision History Date; Version; Author; Description. 2019.12.05; 1.0; 유성엽; 초안 작성. 2019.06.30; 1.1; 유성엽; 초안 작성. 2022.03.09; 1.2; 유성엽; 내용 수정.

| List of Abbreviations and Acronyms |  |  |
| --- | --- | --- |
| Contents | Description |
| EPS | Electrical Power System |
| SOC | State of Charge |
| DOD | Depth of Discharge |
| EoC | End of Charge |
| PCM | Power Conditioning Module |
| PDM | Power Distribution Module |
| PCDU | Power Conditioning and Distribution Unit |
| MPPT | Maximum Power Point Tracker |
| BCR | Battery Charge Regulator |
| SEPIC | Single Ended Primary Inductor Converter |
| LTAN | Local Time Ascending Node |
| LAPSS | Laser Pulsed Solar Simulator |
| Voc | Open Circuit Voltage |
| Isc | Short Circuit Current |
| Pmp | Maximum Power |
| OBC | On Board Computer |
| COM | Communication |
| ETB | Electrical Test Bed |

Contents

1.	Introduction	3

1.1.	Electrical Power Budget	4

1.2.	Electrical Load analysis	4

2.	Methodology	4

2.1.	Component’s Specifications	4

2.2.	MIMAN subsystem component usage by operating modes	5

2.3.	MIMAN power consumption & limit analysis	5

2.3.1.	Power Consumption Calculation	5

2.3.2.	Limit analysis Calculation	5

3.	Result	6

3.1.	Power budget table	6

3.2.	ELA	6

4.	Conclusion	6

5.	Reference	6

6.	Appendix	6

6.1.	MIMAN EPS Component Electrical Power Description	6

6.2.	MIMAN System operation	6

- Introduction

It is the Electrical Power Budget and Electrical Load analysis about MIMAN EPS.

  - Electrical Power Budget

MIMAN EPS power budget is consist of electrical component’s operating power, pcm eff, operating voltage, percentage about 1 period of each operating modes.

  - Electrical Load analysis

Using Power budget’s components, calculate each EPS voltage BUS limit & average power consumption (12.8~16V Unregulated, 5V regulated, 3.3V regulated 12V regulated buses). ELA shows the peak power load of each bus.

- Methodology

  - Component’s Specifications

For Electrical Power Budget and Electrical Load analysis, we must know the component’s electrical specification.

Table 1. Specification about Electrical Components

Also, PCM efficiency(~0.9) using in power budget & ELA.

We use average voltage, voltage Input types, operating descriptions to calculate power budget & ELA.

  - MIMAN subsystem component usage by operating modes

MIMAN has various operating modes and each mode uses different components in operating. To calculate power budget & ELA, we must know component usage of each modes. From scenario description and mission timeline, we can fill in the duty rate about components.

Table 2. Operating time percentage about electrical components of each modes

  - MIMAN power consumption & limit analysis

From operating time and components specifications, we can calculate the Power consumption & peak power of each buses.

    - Power Consumption Calculation

Total average Power Sum(W) is summation of [Pcomponent×Tcomponent÷PCMeff] except EPS and EPS’s [Pcomponent×Tcomponent].

Total average Power Sum with 20% margin is multiply 1.2 at Total average power sum.

Generated Power avg is from EPS power generation, refer the MIM-TR-EPS-004

    - Limit analysis Calculation

Each EPS bus limits are defined by EPS PCDU. (3.3V regulated bus : 13.16W, 5V regulated bus : 19.56W, 12V regulated bus : 11.5W , unregulated bus : 108W)

From components usage at each mode, limit analysis is comparing the peak power of each bus and bus limits. Bus’s peak power is summation of components’ operating power which using corresponding bus. Before summation, if component uses pcm, we must divide operating power by pcm efficiency (~0.9).

Peak power is summation of each buses peak power.

- Result

  - Power budget table

3.1.1 Normal Mode

3.1.2 Mission mode

3.1.1 Communication Mode

3.1.1 Charge Mode

  - ELA

Table 4. Bus Limits & peak Load about each mode

- Reference

- Appendix