Document No. MIM-TR-EPS-005 Document Author Seung-Kwan Jeon Responsible Engineer Geuk-Nam Kim Date 2019.12.12

EPS H/W Design I

MIMAN EPS COTS Selection Guide and Their Specification

Revision History Date; Version; Author; Description. 2019.12.12; 1.0; 전승권; 초안 작성. 2022.03.08; 1.1; 전승권; 내용 수정.

List of Abbreviations and Acronyms

Contents; Description. SOC; State of Charge. DOD; Depth of Discharge. PCM; Power Conditioning Module. PDM; Power Distribution Module. PCDU; Power Conditioning and Distribution Unit. MPPT; Maximum Power Point Tracker. BCR; Battery Charge Regulator. SEPIC; Single Ended Primary Inductor Converter. LTAN; Local Time Ascending Node. EOL; End Of Life. BOL; Birth Of Life. VD; Voltage Domain.

Contents

1	EPS COTS Selection Criteria	5

1.1	Solar Cell	5

1.1.1	EOL Efficiency	5

1.1.2	Heritage	5

1.2	PCDU	5

1.2.1	PCM & PDM 	5

1.3	Battery	5

1.3.1	Capacity	5

1.3.2	Battery Voltage	5

2	MIMAN EPS COTS Selection	6

2.1	Solar Cell	6

2.1.1	Specification	6

2.1.2	Heritage	6

2.2	PCDU	6

2.2.1	Specification	6

2.3	Battery	6

2.3.1	Battery Cell	6

2.3.2	Specification	6

3	Reference	7

4	Appendix	7

- EPS COTS Selection Criteria

MIMAN EPS는 태양전지, PCDU, 배터리로 구성되어 있다. 아래 기준에 따라 하드웨어를 선정했다.

  - Solar Cell

우리는 EOL 효율성과 그 Heritage를 바탕으로 태양전지 COTS를 선정했다.

    - EOL Efficiency

태양전지의 EOL 효율은 높아야 하며, BOL 효율성과 큰 차이가 없어야 한다.

    - Heritage

다른 우주 미션(CubeSat 미션) 또는 선행 미션(CANYVAL X&C)에 대한 사용 이력이 있어야 한다.

  - PCDU

우리는 그 하드웨어가 다른 서브시스템의 전기적 요건에 맞게 전력을 공급할 수 있는지 여부에 기초하여 PCDU COTS를 선택했다.

    - PCM & PDM

PCDU는 서브 시스템의 전기 장치에 적합한 전압 및 전류를 공급해야 함

  - Battery

배터리 용량과 배터리 전압을 기준으로 배터리 COTS를 선택했다.

    - Capacity

배터리의 용량은 미션 요구 조건에 만족해야 한다.

    - Battery Voltage

배터리 셀 구성이 EPS 작동에 적합해야 함.

- MIMAN EPS COTS Selection

  - Solar Cell

MIMAN EPS에서는 AZURE SPACE사의 “TJ Solar Cell 3G30C - Advanced”를 사용한다.

그림 1. TJ Solar Cell 3G30C - Advanced

    - H/W Electrical Specification

| [Electrical Data] | BOL | 2,5E14 | 5E14 |  | 1E15 |
| --- | --- | --- | --- | --- | --- |
| Average Open Circuit Voc [mV] | 2700 | 2616 | 2564 |  | 2522 |
| Average Short Circuit Isc [mA] | 457.0 | 455.5 | 451.6 |  | 440.9 |
| Voltage at max. Power Vmp [mV] | 2411 | 2345 | 2290 |  | 2246 |
| Current at max. Power Imp [mA] | 442.8 | 442.1 | 439.8 |  | 427.5 |
| Average Efficiency ŋbare (1367 W/m2) [%] | 29.5 | 28.6 | 27.8 |  | 26.5 |
| Average Efficiency ŋbare (1353 W/m2) [%] | 29.8 | 28.9 | 28.1 |  | 26.8 |
|  |  |  | @fluence 1MeV [e/cm2] |  |  |
| 표 1. 전기적 특성 - 3G30C – Advanced |  |  |  |  |  |
| [Acceptance Values] |  |  |  |  |  |
| Voltage Vop [mV] |  |  |  | 2350 |  |
| Min. average current Iop avg @ Vop [mA] |  |  |  | 444 |  |
| Min. individual current Iop min @ Vop [mA] |  |  |  | 417 |  |
| 표 2. 허용 수치 - 3G30C – Advanced |  |  |  |  |  |

    - Heritage

선행 임무(CANYVAL-C)에서는 AZURE SPACE의 태양전지를 사용하고 있다. 그래서 이 회사의 태양전지를 사용하는 것에 경험이 있다.

  - PCDU

MIMAN EPS에서는 ISIS 사의 “Compact EPS version 2 - CEPSv2”를 사용한다

그림 2. ISIS CEPSv2

    - H/W Characteristics

- Maximum power point tracking (MPPT) 기능

- H/W 과전압, 과전류 방지 기능 탑재

- 방사선 내구성 향상을 위한 FRAM 기반 MCU

- I2C 통신 사용

- 추가 보드 장착을 통한 전기적 연결 확장 가능

그림 3. ISIS CEPS PCDU Top Connector Pinout

그림 4. ISIS CEPS PCDU Bottom Connector Pinout

      - PCM

- 총 4개의 전압 영역을 제공한다: 3.3 V @ 4 A, 5 V @ 4 A, 12 V(user selectable) @ 4A, Vbat (12.8 – 16 V) @ 4A (VD0: Vbat/ VD1: 3.3 V/ VD2: 5 V/ VD3: 기본 5 V(최대 12 V 조정 가능)/ VD4: 기본 3.3 V(최대 12 V 조정 가능)

- Mother Board – VD0 & VD1 & VD2, Daughter Board – VD0 & VD3 & VD4

- BCR

그림 5. Chain efficiency – 태양 전지판-배터리 Chain Efficiency – type B & C

      - PDM

- 개별 출력 제어 스위치 제공_모든 출력 채널에 대해 제어 스위치를 제공

(총 16의 출력 채널과 그에 따른 제어 스위치 제공)

    - H/W Electrical Specification

| Electrical Specification |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Static (Idle) consumption IU mainboard |  |  |  |  | 90 | mW |  |
| Power output IU mainboard (type A, B and C) | VD0 | Number of output channels | Type A and B |  | 1 | - |
|  |  |  | Type C |  | 1+2 |  |
|  |  | Output voltage | Type A and B |  | 6.5 – 8.0 | V |
|  |  |  | Type C |  | 12.8 – 16.0 |  |
|  | VD1 | Number of output channels |  |  | 4 | - |
|  |  | Output voltage |  |  | 5 | V |
|  |  | Output max. current per channel |  |  | 2 | A |
|  |  | Output max. current total |  |  | 4 | A |
|  | VD2 | Number of output channels |  |  | 4 | - |
|  |  | Output voltage |  |  | 3.3 | V |
|  |  | Output max. current per channel |  |  | 2 | A |
|  |  | Output max. current total |  |  | 4 | A |
| Power output Daughterboard (type C only) | VD3 | Number of output channels |  |  | 4 | - |
|  |  | Output voltage |  |  | Custom (12 V max) Default: 5V | V |
|  |  | Output max. current per channel |  |  | 2 | A |
|  |  | Output max. current total |  |  | 4 | A |
|  | VD4 | Number of output channels |  |  | 1 | - |
|  |  | Output voltage |  |  | Custom (12 V max) Default: 3.3 V | V |
|  |  | Output max. current per channel |  |  | 2 | A |
|  |  | Output max. current total |  |  | 2 | A |
| Solar panel input | Number of MPPTs inputs |  |  | Type A and B | 3 | - |
|  |  |  |  | Type C | 3+2 |  |
|  | Maximum input voltage |  |  | Type A and B | 8.0 | V |
|  |  |  |  | Type C | 16. |  |
|  | Minimum input voltage to start MPPT |  |  |  | 3.5 | V |
|  | Minimum input voltage if another channel’s voltage is sufficient to start MPPTs |  |  |  | 2.0 | V |
|  | Maximum input current per channel |  |  |  | 2 | A |

표 4. Electrical Specification: iEPS Electrical Power System - CEPS

그림 6. Block Diagram: CEPS – ISIS

  - Battery

MIMAN EPS는 CEPS에서 구성되는 기본 배터리를 사용한다

그림 7. ISIS – CEPS 4-cell battery

    - H/W 사양

- 4 직렬 배터리 팩

- 45Wh 용량 (각 베터리 셀 용량: 3.2Ah)

- 히터 제공

| Configurations | Number of cells | 3200 mAh cell (Nominal) |
| --- | --- | --- |
| 4S | 4 | 45 Wh 12.8 - 16 V 3.2 Ah |

| Battery | Cell | Min., Nom., Max. Voltage (absolute maximum) |  | 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Nominal Capacity |  | 2.5, 3.6, 4.2 |  |
|  | Pack | Configuration | Type A | 2 in series | - |
|  |  |  | Type B and C | 4 in series |  |
|  |  | Operating Voltage (hardware limits) | Type A | 5.5 – 8.0 | V |
|  |  |  | Type B and C | 10.9 – 16.0 |  |
|  |  | Operating Voltage (software limits) | Type A | 6.5 – 8.0 | V |
|  |  |  | Type B and C | 12.8 – 16.0 |  |
|  |  | Max Input Current |  | 1.5 | A |
|  |  | Max Output Current |  | 4 | A |
|  |  | Maximum heater power | Type A | 1.2 | W |
|  |  |  | Type B and C | 2.5 |  |

표 5. ISIS – CEPS 4-cell battery

    - PCDU & Battery

호환성 문제를 이유로 ISIS 사의 CEPS PCDU와 배터리(CEPS)를 그대로 선택했다.

- Reference

http://www.azurspace.com/images/006050-01-00_DB_3G30C-Advanced.pdf

https://www.cubesatshop.com/wp-content/uploads/2019/06/ISIS-EPS-Brochure-web.pdf

ISIS Compact EPS version 2-Hardware ICD

https://www.batteryspace.com/prod-specs/NCR18650B.pdf

- Appendix