Document No. MIM-TR-EPS-008 Document Author Seung-Gwon Jeon Responsible Engineer Seung-Gwon Jeon Date 2019.12.12

EPS Component Test I

MIMAN EPS COTS Components Test and Result: Battery, PCDU

Revision History Date; Version; Author; Description. 2019.12.12; 1.0; 전승권; 초안 작성. 2022.03.08; 1.1; 전승권; 내용 수정.

List of Abbreviations and Acronyms

Contents; Description. SOC; State of Charge. DOD; Depth of Discharge. PCM; Power Conditioning Module. PDM; Power Distribution Module. PCDU; Power Conditioning and Distribution Unit. MPPT; Maximum Power Point Tracker. BCR; Battery Charge Regulator. EOF; Electro-Osmotic Flow. IU; Integrated Unit. DB; Daughter Board.

Contents

Contents	4

Checkout before Test	5

Checkout after Test	5

Battery Test	6

1.	Battery Individual Cell Test	6

2.	Battery Pack assembly Test	8

PCDU Test	10

1.	IU test	10

2.	IU DB test	12

APPENDIX A.	14

APPENDIX B.	16

APPENDIX C.	17

Checkout before Test

- At least two or more people perform test together

- Clean up the surroundings before perform test

- Preparation hard copy of test manual and procedure

Checkout after Test

- Power Off (in order of VCC, DC, GND) -> Visual Check -> Disconnect Data Cable / Electrical Wire

- Arrangement Surroundings: Test Object, equipment, heater, by-products

  - Stop the experiment immediately if it does not work in accordance with the experimental procedures and resume the experiment after discussion with the subsystem / system engineer

* This technical report is about battery and PCDU component test conducted by the manufacturer, ISIS

Battery Test

- Battery Individual Cell Test

Test Purpose

- Checking cell capacitance

Test Target

- Battery cell: LIO-10450, LIO-10451, LIO-10452, LIO-10453

Test result

| LIO-10450 | LIO-10451 |
| --- | --- |
| LIO-10452 | LIO-10453 |

< Charge-discharge cycle test for individual cell >

| Description | Expected result | Lio cell number | Result | Unit |
| --- | --- | --- | --- | --- |
| Measured cell capacitance | >2518 mAh | LIO-10450 | 2638 | mAh |
|  |  | LIO-10451 | 2678 | mAh |
|  |  | LIO-10452 | 2678 | mAh |
|  |  | LIO-10453 | 2645 | mAh |

< Individual cell capacitance test result >

- Battery Pack assembly Test

Test Purpose

- Checking of Battery’s Characteristics in discharge – charge cycle

Test Target

- Power battery pack, Individual cell

Test Result

| Description | Expected result |  | Initial Results | Final Results | Unit |
| --- | --- | --- | --- | --- | --- |
| Visual inspection- |  |  |  |  |  |
| Mechanical quality: Correct PCB integrity with no bent pins, no broken components or loose wiring, good quality soldering, no burn marks, residues and major scratches. | All OK |  | All OK | All OK | - |
| Battery pack weight | 248±10 g |  | - | 252 | g |
| Battery pack |  |  |  |  |  |
| Measured cell capacitance | Cycle 1 | >2518 mAh | 2660 | 2652 | mAh |
|  | Cycle 2 | >2518 mAh | 2654 | 2649 | mAh |
|  | Cycle 3 | >2518 mAh | 2589 | 2649 | mAh |
|  | Cycle 4 | >2518 mAh | 2589 | 2649 | mAh |
| Total pack voltage (EOF charge) 30% SOC | 14.7 ± 0.15V |  | Polarity: OK | 14.63 | V |
| Resistance of the battery pack heater | 120Ω ± 10Ω |  | 121.5 | 121.5 | Ω |
| Temperature sensor resistance. | 80kΩ ~ 120kΩ | Temp 1 | 110.6 | 96.8 | kΩ |
|  |  | Temp 2 | 110.5 | 99.0 | kΩ |
| Individual cell voltage | Cell 1 | 3.65±0.15V | Polarity: OK | 3.658 | V |
|  | Cell 2 | 3.65±0.15V | Polarity: OK | 3.659 | V |
|  | Cell 3 | 3.65±0.15V | Polarity: OK | 3.660 | V |
|  | Cell 4 | 3.65±0.15V | Polarity: OK | 3.659 | V |

< Battery pack assembly test result >

| Charge-discharge cycle initial results | Charge-discharge cycle final results |
| --- | --- |

< Charge-discharge cycle test result >

Thermal Cycle

< Thermal cycle test result >

PCDU Test

- IU test

Test Purpose

- Checking EPS board’s channel voltage

- Checking EPS board’s functional operation

Test Target

- PCDU(IU)

Test Result

| Description | Expected result | Initial Results | Final Results |
| --- | --- | --- | --- |
| Visual inspection |  |  |  |
| Mechanical quality: Correct PCB integrity with no bent pins, no broken components or loose wiring, good quality soldering, no burn marks, residues and major scratches | All OK | All OK | All OK |
| Initial test |  |  |  |
| No shorts measured | R > 5kΩ | R > 5kΩ | R > 5kΩ |
| Function of battery and satellite enable |  |  |  |
| Battery enable | OK | OK | OK |
| Satellite enable | OK | OK | OK |
| Voltage of output channels |  |  |  |
| OBC00 - VBAT_perm | UBAT_OBC 00 = 16.0±0.1 V | 15.983 V | 15.983 V |
| OBC01 - 5V_perm | U5V_OBC 01 = 5.0±0.1 V | 5.023 V | 5.027 V |
| OBC05 - 3V3_perm | U3V3_OBC 05 = 3.3±0.1 V | 3.331 V | 3.33V V |
| OBC02 - 5V_sw1 | U5V_OBC 02 = 5.0±0.1 V | 5.023 V | 5.026 V |
| OBC03 - 5V_sw2 | U5V_OBC 03 = 5.0±0.1 V | 5.032 V | 5.027 V |
| OBC04 - 5V_sw3 | U5V_OBC 04 = 5.0±0.1 V | 5.032 V | 5.026 V |
| OBC06 - 3V3_sw1 | U3V3_OBC 06 = 3.3±0.1 V | 3.332 V | 3.330 V |
| OBC07 - 3V3_sw2 | U3V3_OBC 07 = 3.3±0.1 V | 3.333 V | 3.333 V |
| OBC08 - 3V3_sw3 | U3V3_OBC 08 = 3.3±0.1 V | 3.330 V | 3.332 V |
| Overcurrent protection output channels |  |  |  |
| Overcurrent Protection - Channel 0: OBC00 - VBAT_perm | UVBAT_perm = 15.3±0.3 V | 15.0 V | 15.0 V |
|  | IOVERCURRENT = 3.0±0.1 A | 3.0 A | 3.0 A |
| Overcurrent Protection - Channel 1: OBC01 - 5V_perm | U5V_perm= 4.7±0.3 V | 4.8 V | 4.8 V |
|  | IOVERCURRENT = 2.9±0.1 A | 2.9 A | 2.9 A |
| Overcurrent Protection - Channel 5: OBC05 - 3V3_perm | U3V_perm= 3.0±0.3 V | 3.1 V | 3.1 V |
|  | IOVERCURRENT = 2.9±0.1 A | 2.9 A | 2.9 A |
| Overcurrent Protection - Channel 2: OBC01 - 5V_sw1 | U5V_SW1= 4.7±0.3 V | 4.7 V | 4.8 V |
|  | IOVERCURRENT = 2.9±0.1 A | 2.9 A | 2.9 A |
| Overcurrent Protection - Channel 3: OBC02 - 5V_sw2 | U5V_SW2= 4.7±0.3 V | 4.7 V | 4.8 V |
|  | IOVERCURRENT = 2.9±0.1 A | 2.9 A | 2.9 A |
| Overcurrent Protection - Channel 4: OBC03 - 5V_sw3 | U5V_SW3= 4.7±0.3 V | 4.7 V | 4.8 V |
|  | IOVERCURRENT = 2.9±0.1 A | 2.9 A | 2.9 A |
| Overcurrent Protection - Channel 6: OBC06 – 3V3_sw1 | U3V3_SW1= 4.7±0.3 V | 3.1 V | 3.1 V |
|  | IOVERCURRENT = 2.9±0.1 A | 2.9 A | 2.9 A |
| Overcurrent Protection - Channel 7: OBC07 – 3V3_sw2 | U3V3_SW2= 4.7±0.3 V | 3.1 V | 3.1 V |
|  | IOVERCURRENT = 2.9±0.1 A | 2.9 A | 2.9 A |
| Overcurrent Protection - Channel 8: OBC08 – 3V3_sw3 | U3V3_SW3= 4.7±0.3 V | 3.1 V | 3.1 V |
|  | IOVERCURRENT = 2.9±0.1 A | 2.9 A | 2.9 A |
| System overcurrent protection |  |  |  |
| System Overcurrent Protection | ISYS.OVERCURR = 4.0±0.1 A | 4.042 A | 4.057 A |
| Heater control |  |  |  |
| Heaters Activation Range | THEATER.ON = 2±1 °C | 2°C | 2°C |
|  | THEATER.OFF = 6±1 °C | 6°C | 6°C |
| Current Consumption Heaters | IHEATERS = 255±10 Ma | 257 mA | 250 mA |
| MPPT |  |  |  |
| MPPT channel 0 | OK | OK | OK |
| MPPT channel 1 | OK | OK | OK |
| MPPT channel 2 | OK | OK | OK |
| Undervoltage Protection |  |  |  |
| Voltage above undervoltage protection | USYSTEM= 11.7±0.1 V | 11.8 V | 11.8 V |
| Voltage below undervoltage protection | USYSTEM= 10.3±0.1 V | 10.3 V | 10.3 V |

< IU system test result >

- IU DB test

Test Purpose

- Checking EPS daughter board’s channel voltage

- Checking EPS daughter board’s functional operation

Test Target

- PCDU(IU-DB)

Test Result

| Description | Expected result | Initial Results | Final Results |
| --- | --- | --- | --- |
| Visual inspection |  |  |  |
| Mechanical quality: Correct PCB integrity with no bent pins, no broken components or loose wiring, good quality soldering, no burn marks, residues and major scratches | All OK | All OK | All OK |
| Initial test |  |  |  |
| No shorts measured | R > 5kΩ | R > 5kΩ | R > 5kΩ |
| Function of battery and satellite enable |  |  |  |
| Battery enable | OK | OK | OK |
| Satellite enable | OK | OK | OK |
| Voltage of output channels |  |  |  |
| VD0_SW[1] | UBAT_OBC 09 = 16.0±0.1 V | 16.0 V | 16.0 V |
| VD0_SW[2] | U16V_OBC 10 = 16.0±0.1 V | 16.0 V | 16.0 V |
| VD0_SW[3] | U16V_OBC 11 = 16.0±0.1 V | 16.0 V | 16.0 V |
| VD3_SW[0] | U5V_OBC 02 = 5.0±0.1 V | 5.0 V | 5.0 V |
| VD3_SW[1] | U5V_OBC 03 = 5.0±0.1 V | 5.0 V | 5.0 V |
| VD4_SW[0] | U5V_OBC 04 = 12.0±0.1 V | 12.0 V | 12.0 V |
| VD4_SW[1] | U3V3_OBC 06 = 12.0±0.1 V | 12.0 V | 12.0 V |
| VD5_SW[0] | U28V2_OBC 16 = 28.2±0.1 V | 28.2 V | 28.2 V |
| Overcurrent protection output channels |  |  |  |
| Overcurrent Protection - Channel 9: VD0_SW[1] | UVD0_SW[1] = 15.3±0.3 V | 15.1 V | 15.1 V |
|  | IOVERCURRENT = 3.15±0.15 A | 3.2 A | 3.2 A |
| Overcurrent Protection - Channel 10: VD0_SW[2] | UVD0_SW[2]= 15.3±0.4 V | 15.1 V | 15.0 V |
|  | IOVERCURRENT = 3.15±0.15 A | 3.2 A | 3.2 A |
| Overcurrent Protection - Channel 11: VD0_SW[3] | UVD0_SW[3]= 15.3±0.4 V | 15.0 V | 15.0 V |
|  | IOVERCURRENT = 3.15±0.15 A | 3.3 A | 3.3 A |
| Overcurrent Protection - Channel 12: VD3_SW[0] | UVD3_SW[0]= 4.7±0.3 V | 4.7 V | 4.9 V |
|  | IOVERCURRENT = 3.15±0.15 A | 3.2 A | 3.2 A |
| Overcurrent Protection - Channel 13: VD3_SW[1] | UVD3_SW[1]= 4.7±0.3 V | 4.7 V | 4.8 V |
|  | IOVERCURRENT = 3.15±0.15 A | 3.2 A | 3.2 A |
| Overcurrent Protection - Channel 14: VD4_SW[0] | UVD4_SW[0]= 12.0±0.3 V | 11.7 V | 12.0 V |
|  | IOVERCURRENT = 3.15±0.15 A | 3.2 A | 3.2 A |
| Overcurrent Protection - Channel 15: VD4_SW[1] | UVD4_SW[1]= 12.0±0.3 V | 11.7 V | 11.9 V |
|  | IOVERCURRENT = 3.15±0.15 A | 3.2 A | 3.2 A |
| Overcurrent Protection - Channel 16: VD5_SW[0] | UVD5_SW[0]= 28.0±0.3 V | 28.1 V | 28.3 V |
|  | IOVERCURRENT = 1.5±0.1 A | 1.5 A | 1.5 A |
| MPPT |  |  |  |
| MPPT channel 3 | OK | OK | OK |
| MPPT channel 4 | OK | OK | OK |

< IU-DB system test result >

APPENDIX A.

- [PCDU (IU) & Battery Pack Connection]

< Type C (Motherboard + Daughterboard)+ Battery Pack(4 cell) >

- [PCDU (IU) & Battery Pack Flight model]

| Battery pack top view | Battery pack bottom view |
| --- | --- |
| Battery pack front view | Battery pack back view |

< Battery Pack(4 cell) Flight Model >

| PCDU Top View | PCDU Bottom View |
| --- | --- |

APPENDIX B.

- [ISIS Compact EPS block diagram]

< ISIS Compact EPS version 2 block diagram >

APPENDIX C.

- [Test result graph]

< Charge-discharge cycle initial results >

< Charge-discharge cycle final results >

< Charge-discharge cycle test for individual cell ; LIO-10450>

< Charge-discharge cycle test for individual cell; LIO-10451>

< Charge-discharge cycle test for individual cell; LIO-10452>

< Charge-discharge cycle test for individual cell; LIO-10453>