Astrodynamics 1

Exercise

1. In an inertial coordinate system, the position and velocity vectors of a satellite are, respectively, (4.1852 I + 6.2778 J + 10.463 K) 107 ft and (2.5936 I + 5.1872 J) 104 ft/sec where I, J and K are unit vectors. Determine the specific mechanical energy, ε, and the specific angular momentum, h.  Also, find the flight-path angle, Φ.

> specific mechanical energy = 1.573*10^9 ft^2/sec^2

> specific angular momentum = 6.0922*10^12 ft^2/sec

> flight-path angle = 35.42 deg

2. For a given satellite, ε = - 2.0x108 ft2/sec2 and e = 0.2.  Determine its specific angular momentum, semi-latus rectum, and semi-major axis.

> specific angular momentum = 6.897*10^11 ft^2/sec

> semi-latus rectum = 3.3790*10^7 ft

> semi-major axis = 3.5198*10^7 ft

3. A radar tracking station tells us that a certain decaying weather satellite has e = 0.1 and perigee altitude = 200 n . mi. Determine its altitude at apogee, specific mechanical energy, and specific angular momentum.

> altitude of apogee = 1009.8 n.mi

> specific mechanical energy = -2.861*10^8 ft^2/sec^2

> specific angular momentum = 5.855*10^11 ft^2/sec

4. A space probe is to be launched on an escape trajectory from a circular parking orbit which is at an altitude of 100 n mi above the earth. Calculate the minimum escape speed required to escape from the parking orbit altitude. (Ignore the gravitational forces of the sun and other planets.) Sketch the escape trajectory and the circular parking orbit.

> Minimum escape speed = 36157.9 ft/s

5. The position and velocity of a satellite at a given instant are described by

r = 2I + 2J + 2K (Distance Units),

v = -0.4I + 0.2J + 0.4K (Distance Units per Time Unit)

where I J K is a nonrotating geocentric coordinate system. Find the specific angular momentum and total specific mechanical energy of the satellite.

> h_vec = 0.4I -1.6J+1.2K (DU^2/TU)

> specific mechanical energy = -0.1087 (DU^2/TU^2)

6. An earth satellite is observed to have a height of perigee of 100 n mi and a height of apogee of 600 n mi. Find the period of the orbit.

> P = 5861 sec

7. Identify each of the following trajectories as either circular, elliptical, hyperbolic, or parabolic (b):

a.  r = 3 DU

v = 1.5 DU/TU  > Hyperbola

b. rperigee= 1.5 DU

p = 3 DU          > Parabola

c. ε = -1/3 DU2/TU2

p = 1.5 D U   > Circle

d.  r = J + 0.2K

v = 0. 9I + 0.123K

e.  r = 1.01K

v = I + l.4K